# Thermal simulations STL and Paraview

Computational Fluid Dynamic (CFD) simulations in general divide geometry up into smaller blocks and calculate energy and mass balance at the borders. Increasing the cell count, together with increasing the order of applied calculation schemes increases accuracy. Both measures increase the computational requirements too.

## Available simulation cases:

| name | experiment | No. people | heated glass | floorheating | Air Exchange Rate |
| --- | --- | --- | --- | --- | --- |
|  | n | n | T [C] | T [C] | n |
| STL_IGU_55 | 1 | 0 | 55 | 20 | 1 |
| STL_IGU_28 | 2 | 0 | 28 | 20 | 1 |
| STL_IGU_25_FH_25 | 3 | 0 | 25 | 25 | 1 |
| STL_IGU_55_N1_AER.5 | 4 | 1 | 55 | 20 | 0.5 |
| STL_IGU_28_N2_AER1 | 5 | 2 | 28 | 20 | 1 |
| STL_IGU_25_N4_AER2 | 6 | 4 | 25 | 25 | 2 |
| STL_IGU_28_N6_AER4 | 7 | 6 | 28 | 20 | 4 |
| STL_IGU_55_N6_AER4 | 8 | 6 | 55 | 20 | 4 |

## Boundary conditions and simulation setup:

| Temperature | [K] | [C] |
| --- | --- | --- |
| Tambient | 278.15 | 5 |
| T Technical space | 293.15 | 20 |
| T other testroom | 293.15 | 20 |
| T start | 293.15 | 20 |
| T inlet | 291.15 | 18 |
| skin temperature | 306.95 | 33.8 |
| Heat output person | 130W |  |

- k-epsilon turbulence model. Ideal gas, compressible.

- externalWallHeatFluxTemperature heat transfer

  - coefficient mode for wall temperature BC

  - power mode for mannequin temperature BC,

- simplefoam transient state

- P1 radiation model

- 1e-4 residuals on: P_rgh, U, h, k, epsilon

- 1e-5 on G

- dt = 1s.

See the title page for geometry indication, the same data can be visualized in Paraview.

## Why Paraview

Because analyzing such vast 3D-datasets is common in multiple fields, there is a good use for a program like Paraview https://www.paraview.org/ . This program allows you to analyze fields (U, P, T, Q, G) or any arbitrary data-format with a suitable reader.

In this document I will walk you through my workflow for Paraview, this comprises of the following:

- Starting up an analysis by opening a suitable file

  - I do this while the simulation is running, this allows me to refresh files and see the latest results as they are written

- Analyze the geometry

- Make suitable cuts in the 3D-geometry to visualize the results in 2D

- Analyze the different available fields and check for anomalies

- Create plots and export tabular data/screenshots

## Installation

Installation of ParaView is straightforward, go to the following site: https://www.paraview.org/download/ Download the Paraview-x.xx.x-MPI executable version and install this.

# Tutorial

Some steps done below can be quickly done by loading the .pvsm files with the Load state button, the files are slightly different.

The files for this tutorial should be downloaded (at least one case from the table above)

## Opening paraview

After installation, open Paraview, and start with going to File -> open (red) and open the STL_IGU.foam (red) file (this activates the openfoam reader built-in into Paraview)

You can see the numbered folders in green, these represent simulation steps, if a transient solver was used, these would be timesteps, in this case they show different states of flow and temperature development. In general it is better to study the later simulation steps for transient simulations.

On the left in the red box you see the pipeline browser, here you see all operations represented as a tree structure. This allows you to setup complex data processing pipelines parallel from eachother.

Below that you see the properties box, you’ll spend a lot of time here setting up visualizations. Here it shows what mesh regions to show, starting with internalMesh is the best Idea, so press apply to start loading the simulation.

At first this doesn’t tell a lot, and immediately brings the problem of 3D simulations to light, data for each point in space.

In the green box above you see the field indicator, here you can select the speed field (u), pressure (p), radiation (G, W/m2), temperature (T) and more. You could calculate new fields with postprocessing steps in ParaView (for example thermal comfort, PMV, etc). Select the Temperature field T for now.

In the yellow box you see the time indicator, you can go to the latest timestep available for now.

Now select STL_IGU.foam (blue) in the pipeline browser and press <ctrl+space>, a pop-up search-window opens to tie new functions to your current selection. Now type slice and click the symbol (blue) and press apply in the properties. Now you get some insight into the simulation.

Now Make a similar cut for the Y-plane by opening the search-window and creating a new slice (select STL_IGU.foam first). In the properties browser you get access to more settings by pressing the cogwheel, and you can quickly select the Y-plane as normal for cutting.

Now select Slice1 and open the search-window, look for Contour and go to properties.

Now select contour by: T and clear the value range with the cross. Now make a new range with the range button from for example 290 – 305 in 25 steps.

Select the file again and look for the clip function. In properties select clip type scalar and select Scalars  T. Now untick the invert tickbox, and you can visualize all temperatures above the temperature set with the value slider.

Now you can set a better color scheme to for example determine zoning by playing with the scale, see below screenshots.

Another valuable function for characterization is the plot over line and plot over time function. Here I’ll  show the first. Select the file and search for plot over line and apply in properties.

A plot shows, and as shown above 6 coordinates for 2 points can be set, the start and end-point of your line. Values can be retrieved along this line. If you scroll down, you can filter out fields to plot.

Another important trick is the extensive export functions paraview offers, select anything and press the screenshot buttons in the windows with CTRL/SHIFT modifier keys and you can save screenshots or tabular data immediately, or through the file menu after selection.

Next to that you can split windows and open for example the spreadsheet view, this allows you to view any compatible data as a table and export it as such. If you can’t export it conveniently yet, see if you can transform it with Paraview, it’s fast and efficient and easy to work with after you get used to it.

## Final exercise – compare different simulations

The next goal will be to load multiple simulations in ParaView and compare them in the same view.

Start with adding a Z-normal slice to the original file, hide all other slices and operations by clicking the eyeball symbol, and center to the X-Y-plane with the button in the toolbar. Also close the other views and resize the color-legend.

Now rename the slice to represent your simulation, and open an additional *.foam file in the pipeline browser (through file menu).

In the properties you can see which simulation folder is loaded, make sure they are different.

First we need to slice the newly loaded file the same. Select the new STL_IGU.foam and make a Z-normal slice. Now we need to translate the slice up or left to not block the current visualization. In the properties browser select the latest slice, look for tran and enter a -3.5 y translation. This will place it just below the previous cut.

These steps can be repeated for all simulations and different slice directions, comparisons between different situations can be made. By playing with the scale and representation valuable insights can be made. This shows a scale from 290 to 301 K, which indicates the difference between a 28C pane and 55C pane well at this specific slice location.

## Load state

Load state is a handy feature to load a pre-set arrangement of windows and analysis, it also restores any tabs you created. Press file -> load state and find a STL_Tut.pvsm or STL_ThermalAnalysis.pvsm  file and press ok.  Then select search under specified folder in the pop-up and redirect to the *.foam file of the case you want to load.